Lab 3：Fourier Series Representation of Periodic Signals

| Author | Name：袁通 代育津 Student ID: 11810818 11910114 |
| --- | --- |
| Introduction In this lab, we will explore the Fourier Series representation of periodic signals. We have totally four problems in this lab: In problem 3.5, we will first synthesize a periodic discrete-time signal, then examine the DTFS representation of several different square waves and finally write a function which computes the DTFS coefficients of a periodic signal. Lab results & Analysis： 3.5 Synthesizing Signals with the Discrete-Time Fourier Series From the coefficient given, we found that a0 is real, and a2 and a-2, a4 and a-4 are conjecture with each other, so the imaginary part will offset each other, so x[n] is real. For N=5, we can derive that, a1=a-4=2e-jπ3, a3=a-2=e-jπ/4 , so here we have a=[1, 2e-jπ3, ejπ4, e-jπ4, 2ejπ3] We define x=[0 1 2 3 4] and we get the plot, the real and imaginary part of signal, from the plot we can see that signal x[n] has only nonzero real part, so our prediction is verified. Here the plots of three signals are shown below. The plots of DTFS coefficients of three signals are shown below. We can derive that a0=1Tx=-T2T2x[n], so for x1[n] a1=88=1 for x2[n] a1=816=0.5 for x3n a1=832=0.25, and we can extract the same result from the plot, which also verify our predict. The signal which fewer coefficient synthesized are shown in the plot, we found that with more coefficient, the synthesized signal is more similar to the original signal xn. Form the plot of real and imaginary part of the signal we can found that the imaginary part of x3_all is 0 so the signal is real. The Gibb’s phenomenon is the peculiar manner in which the Fourier series of a piecewise continuously differentiable periodic function behaves at a jump discontinuity. From the plots we can find several significantly higher point before the signal jump from 1 to 0, and several significantly lower point after the jump, which is called the Gibb’s phenomenon. We can also find that the range of the phenomenon is much wider when we synthesized less coefficient, and if we let more coefficient involved the phenomenon is hard to identify due to the low accuracy of the plot. function a=dtfs(x,n_init); a=[]; w=2*pi/length(x); %fundamental frequency for k=n_init:n_init+length(x)-1 %period from 0+n0 to N-1+n0 a_k=0; for n=1:length(x) a_k=a_k+x(n)*exp(-j*k*w*(n+n_init-1)); end a=[a a_k/length(x)]; end if n_init<0 for i=1:-n_init a=[a a(i)]; end a=a(1-n_init:length(a)); else if n_init>0 for i=i:n_init a=[a(length(a)-i+1) a]; end a=a(1:length(x)); end end 3.8 First-Order Recursive Discrete-Time Filters Note: Please indicate meaning of the symbols in all expressions. Please indicate the coordinate and unit in all figures. |  |
| Experience You can write your experience with this project. Any comment and suggestion on this course are also very welcome. |  |
| Score |  |

字体：英文Times new Roman；中文宋体，正文五号

文件名统一命名方式：LabX+姓名+学号，例如：Lab1+张三+00001 （正式报告删除此行！）